Parking and Passenger Loading Zones

|  | Required | Actual |  |
| --- | --- | --- | --- |
| Accessible Parking |  |  |  |
| Number of spaces | 1 van accessible space for every 8 total spaces | Total Number of spaces: | # of Accessible Spaces: |
| Location | Closest to accessible entrance on accessible route |  |  |
| Size of accessible space | 96” wide |  |  |
| Access aisle | 60” wide for car; 96” wide for van spaces Note: Two spaces can share one aisle |  |  |
| Slope and cross-slope | Maximum slope of 2%. |  |  |
| Signage | International symbol of accessibility Not obscured by parked vehicle |  |  |
| Surface | Firm, stable, and slip-resistant |  |  |
| PASSENGER LOADING ZONE |  |  |  |
| Location | Access aisle adjacent and parallel with loading zone |  |  |
| Size of Aisle | Access aisle at least 60: x 20’ |  |  |
| Slope and cross slope | Slope/cross-slope max 2% |  |  |

Accessible Route

|  | Required | Actual |
| --- | --- | --- |
| Number | Must be at least one accessible route from public transportation stops, parking, street, sidewalk to the building they serve |  |
| Width | Min width must be 36” (except at doors which must be 32” wide) |  |
| Surface | Firm, stable and slip-resistant Carpet should be low-pile (1/2” at most) |  |
| Slope | Max 5%. If it’s more than5%, it’s a ramp |  |
| Cross-slope | Max 2% |  |
| Changes in level | ¼”-1/2” should be beveled more than 1/2” should be ramped |  |
| Head Room | Min. 80” clear space |  |
| Protruding Objects | Leading edge 27-80” AFF, protrude no more than 4” Leading edge below 27” may protrude any amount |  |
| Grates | If in the direction of travel, openings no more than ½” wide. If elongated openings, perpendicular to direction of travel |  |

Curb Ramps

|  | Required | Actual |
| --- | --- | --- |
| Slope | Slope max 8.33% |  |
| Diagonal (Corner) Curb Ramps | Min. 36” wide (excluding flared sides) If pedestrians must walk across it, must have flared sides with less than 10% slope ADA requires detectable warnings and visual contrast |  |

Ramps

|  | Required | Actual |
| --- | --- | --- |
| Width | At least 36” |  |
| Rise | Max 30” |  |
| Slope | 5%-8.33% |  |
| Cross-slope | Max 2% |  |
| Surface | Firm, stable, and slip resistant; designed so water won’t accumulate |  |
| Landings | At top and bottom of ramp. Should be level and at least as wide as ramp; Level landings at each change in direction a min. of 60” x 60” |  |
| Drop-offs | Must be protected by wall, railing or 2” curb |  |
| Handrails | Required on both sides when ramp rises more than 6” or projects more than 72” Diameter 1-1/4” to 1-1/2” 1-1/2 “ between handrail and adjacent wall Extend 12” beyond the top and bottom of ramp Top of gripping surface 30-34” AFF (ADA:34-38”) Should be solidly anchored and not rotate in fittings |  |

Entrances and Doors

|  | Required | Actual |
| --- | --- | --- |
| Entrances |  |  |
| Location | On accessible route from parking/public transportation Directional signage at inaccessible entrances and decision points |  |
| Doors |  |  |
| Type | Standard single or double-leaf hinged doors, no revolving doors or turnstiles If double doors, at least one must comply |  |
| Opening | Clear opening with width min 32”, with the door open 90 degrees |  |
| Door w/closer | At least 3 seconds to close |  |
| Door hardware | Lever or push/pull type, no tight-grasping, twisting or pinching of the wrist and operated with one hand Mounted max 48” |  |
| Thresholds | Not more than ½” Raised thresholds must be beveled |  |
| Door opening Force | Exterior doors: no standard Interior doors: max 5 lbs |  |
| Automatic/ Power-Assisted Doors | AT least 3 seconds to open |  |

Assembly Areas

|  | Required | Actual |
| --- | --- | --- |
| Route | Located on an accessible route Accessible route should connect wheel chair locations w/performing areas (stage, arena floors, locker rooms). |  |
| Door | Min 32” clear opening Hardware must be lever or push/pull type w/no twisting or tight grasping Hardware mounted max 48” Max 5 lbs. opening force |  |
| Thresholds | Not more than 1/2” Raised thresholds must be beveled |  |
| Flooring | Firm, stable and slip-resistant Carpet is low-pile and securely attached |  |
| Accessible seating location |  |  |
| Number | At least 5% of the seating capacity, but not less than one. ADA requires at least one companion seat | Seating capacity: # of wheel chair locations: # of companion seats: |
| Location | Wheel chair locations should be dispersed, provide comparable lines of sight, and be located on accessible route |  |
| Route | There should be an accessible route to and through the seating area |  |
| Clear floor space | At least 30” x 48” allowing for front or parallel approach Shouldn’t overlap knee space by more than 19”. |  |
| Knee clearance | Must be at least 27” high 30” wide, 19” deep |  |
| Height | 28-34” AFF to top of table |  |
| Assistive listening systems |  |  |
|  | If have audio-amplification system, must have a listening system to serve at least 2 seats; If serves individual fixed seats, then seats must be with in 50’ of stage. ADA: 4% of total number of seats but not less than2 plus signage indicating availability of system |  |
| Alarms |  |  |
|  | UFAS: If provided must have both visible and audible signals ADA: requires in areas of general usage |  |

Public Restrooms

|  | Required | Actual |
| --- | --- | --- |
| Number | If public restrooms are provided, at least one must be accessible and on an accessible route |  |
| Door | Min. 32” clear opening Shouldn’t swing into the CFS for any fixture Hardware must be lever or push/pull type w/no twisting or tight grasping Hardware mounted max 48” Max 5lbs opening force |  |
| Toilet In stall |  |  |
| Toilet stall opening | At least 32” wide |  |
| Stall size | Wall mounted-toilet: 56” deep x 60 “ wide Both measured facing the toilet |  |
| Toilet | Seat 17”-19” AFF Mounted exactly 18” from center of toilet to closest side wall |  |
| Toilet paper dispenser | Mounted w/in reach and min. 19” AFF to centerline of the toilet paper Allows continuous paper delivery |  |
| Flush control | Max 44” AFF on wide side of toilet area Automatic or operable w/one hand. No tight grasping or twisting of the wrist. |  |
| Grab bars | Required on back and side of toilet Back: Min 36” long, starting no more than 6” from side wall Side: min 40” long, starting no more than 12” from back wall. Mounted 33”-36’ AFF 1-1/4 -1-1/2 “ diameter 1-1/2 “ from wall |  |
| Crutch stall | Need one if have 6 stalls Has parallel grab bars, door swings out |  |
| (Public Restroom Table continued on next page) |  |  |

| (Public Restroom Table continued from previous page) |  |  |
| --- | --- | --- |
|  | Required | Actual |
| Toilet not in stall |  |  |
| Toilet | Seat 17”-19” AFF Mounted exactly 18” from center of toilet to closest side wall |  |
| Toilet paper dispenser | Mounted w/in reach and min 19” AFF to centerline of the toilet paper Allows continuous paper delivery |  |
| Flush control | Max 44” AFF On wide side of toilet area Automatic or operable w/one hand, no tight grasping or twisting of the wrist |  |
| Grab bars | Required on back and side of toilet Back: min 36” long, w/one end mounted at least 12” from centerline of toilet Side: min 42” long, w/front end a minimum 54” from back wall Mounted 22”-36” AFF 1-1/4 “ – 1-1/2” diameter 1-1/2” from wall |  |
| Urinals |  |  |
| Height | Elongated rim and max 17” AFF |  |
| Clear floor space | 30” x 48 “ for forward approach |  |
| Flush control | Max 44” AFF Automatic or operable w/one hand. No tight grasping or twisting of the wrist |  |
| Lavatory (sink) |  |  |
| Height | Rim or countertop max of 34” AFF |  |
| Knee space | Min 29” clearance to bottom of the apron |  |
| Clear floor space | 30” x 48 “ for forward approach |  |
| Faucet controls: | Hand operated or automatic Do not require tight gripping or twisting of the wrist. |  |
| Pipes | Drain and hot water pipes should be insulated |  |
| Other elements |  |  |
| Mirrors | Mounted w/bottom edge of reflecting surface max 40” AFF |  |
| Dispensers Paper towels Trash receptacle Coat hooks Feminine Hygiene, etc. . . | 30” x 48 “ for forward or parallel approach; highest operable part can’t exceed 48” for forward approach 54” for side approach Operable w/one hand; no tight grasping or twisting of the wrist |  |
| Alarms | If provided, must have both visible and audible signals. ADA requires visible alarms in restrooms and general usage areas, hallways, lobbies, and any other area of general use |  |

Drinking Fountains

|  | Required | Actual |
| --- | --- | --- |
| Minimum number | ADA: If there’s more than1 on a floor, 50% must be accessible. If only one on a floor, there should be a “hi-lo fountain” UFAS: 50% on each floor must be accessible to persons in wheelchairs |  |
| Free-standing or built-in units | CRS at least 30” x 48” to allow parallel approach |  |
| Wall- and post-mounted units | Need clear knee space: 27” from floor to bottom of apron; 30” side and 17-19” deep to wall CFS of at least 30” x 48” to allow fro forward approach |  |
| Spout | Max 36” from floor to spout outlet Located at front of unit |  |
| Controls | Front-or side-mounted near front edge Operable with one hand with no tight grasping, pinching or twisting of the wrist |  |

Telephones

|  | Required | Actual |
| --- | --- | --- |
| Number required | At least one accessible telephone must be provided at each bank of telephones |  |
| Clear floor/ground space | At least 30” x 48 “ for either forward or parallel approach |  |
| Hearing aid compatible and volume control | Telephones must be hearing aid compatible Volume control: UFAS: required on at least one phone ADA: all accessible phones and 25% of all others w/signage |  |
| Telephone cord length | Min 29” long |  |
| Telephone books | Highest operable part max 48” for forward approach Max 54” for parallel approach |  |
| Text telephones (TTD/TTY) | UFAS doesn’t require ADA requires one if there’s four or more public phones in a building. Must have TDD sign and directional signage |  |

Elevator

|  | Required | Actual |
| --- | --- | --- |
| Freight elevators | Freight elevators not acceptable unless the only elevators provided are a combo passenger/freight |  |
| Automatic operation | Elevator operation must be automatic |  |
| Hall Call buttons | Buttons should be centered 42” AFF “up” button should be on top Should have visual signal to show when call is registered and answered |  |
| Hall lanterns | Should have visible and audible signal showing which car is answering a call Audible signals should ring once for up and twice for down or say “up” or down” |  |
| Raised characters on hoist-way entrances | Must have raised floor designations provided on both floor jambs Centered on 60” AFF (ADA requires Braille as well) |  |
| Door protective and reopening device | Door should automatically reopen if it becomes obstructed by and object or person |  |
| Floor plan of elevator cars | Door opening should be 36” Centered opening—51” x 80” Off-center opening—51” x 68” |  |
| Inside call buttons | Raised or flush buttons no higher than 48” and ¾” diameter Located on front all ADA—48 “ for forward reach, 54” for side reach. Button for main floor should have raised star to the left |  |
| Emergency controls/emergency communications | Controls grouped at bottom of panel but not less than 35” AFF Communications max 48” but no less than 15” AFF—should not require voice communication |  |

Fixed Seating and Tables, Storage and controls

|  | Required | Actual |
| --- | --- | --- |
| Fixed seating and tables—including food service counters |  |  |
| Number | At least 5% but not less than 1 |  |
| Route | There should be an accessible rout to and through the seating area |  |
| Clear floor space | At least 30” x 48” allowing for front or parallel approach Shouldn’t overlap knee space by more than 19” |  |
| Knee clearance | Must be at least 27” high, 30” wide, 19” deep |  |
| Height | 28-34” AFF to top of table |  |
| Storage |  |  |
| Number | If provided, in an accessible facility, at least one should be accessible |  |
| Route | There should be an accessible rout to and through the area |  |
| Clear floor space | At least 30” x 48” allowing for front or parallel approach |  |
| Reach | Max 48” for front approach Max 54” for side approach |  |
| Hardware | Operable with one hand, no tight grasping or twisting of the wrist |  |
| Controls |  |  |
|  | All controls should be located 15-48” for forward reach 9-54” for side reach Operable with out tight grasping or twisting of the wrist |  |